# BL21DE3 Competent Cells

BL21 DE3 Competent Cells

Cat # GBBL-2:  20 x 50µl

Cat# GBBL-2-50: 50x50ul

Cat# GBBL-2-96: 96x50ul

Description:

Bl21DE3  is one of the most commonly used E. coli strain for protein expression. High efficiency BL21DE3 chemically competent cells are prepared with unique salt compositions and procedures that result in significantly higher transformation efficiency and stability  than those by traditional lab protocols using CaCl2 buffers.  Biopioneer’s competent cells are quality controlled by direct comparison to other leading brands of similar products in transformation of pUC18 plasmid DNA .

Features:

- Widely used strain for protein expression

- Efficiency: Constantly above 1 x 107 cfu/µg of supercoiled pUC18 plasmid DNA; higher colony counts than BL21DE3 Competent Cells from other leading suppliers.

- Conveniently packaged in 50ul aliquot, no repeated freezing in dry ice and alcohol bath.

Genotype:

Genotype - F-, ompT, hsdSB(rB-mB-), dcm, gal, λ(DE3), tonA

Storage:

It has been observed that long term storage of competent cells at -80oC can actually cause the cells to adapt by increasing the recombination rate. Therefore, BioPioneer Inc. prepares its competent cells frequently at small scales. It is recommended that competent cells be used within 3 months after arrival.

Transformation Protocol

1. Thaw competent cells on ice. It is essential that cells be completely thawed before pipetting.

2. Transfer 50ul cells to a prechilled Falcon tube, then add DNA of your choice to the cells and incubate on ice for 30 minutes.

3. Heat-shock at 42°C for 30 seconds and put the tube on ice for 5 minutes.

4. Add 300ul SOC or LB broth and shake the cells at 37°C at 225 rpm for 30-60 minutes.

5. Plate out desired amount on a pre-warmed (37°C) plate with the appropriate antibiotic and incubate in a 37oC incubator.

Warranty

These products are warranted to perform as described in their labeling and in BioPioneer literature when used in accordance with their instructions. THERE ARE NO WARRANTIES THAT EXTEND BEYOND THIS EXPRESSED WARRANTY AND BIOPIONEER DISCLAIMS ANY IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR PARTICULAR PURPOSE. BioPioneer’s sole obligation and purchaser’s exclusive remedy for breach of this warranty shall be, at the option of BioPioneer Inc., to repair or replace the products. In no event shall BioPioneer Inc.  be liable for any proximate, incidental or consequential damages in connection with the products.