CellMAXTM LDH-Cytotoxicity Assay Kit

Catalog # : CT-001, 500 assays; Store kit at –20oC

I. Introduction:

Cell death or cytotoxicity is classically evaluated by the quantification of plasma membrane

damage. Lactate dehydragenase (LDH) is a stable enzyme, present in all cell types, and

rapidly released into the cell culture medium upon damage of the plasma membrane. LDH,

therefore, is the most widely used marker in cytotoxicity study. BioPioneer’s CellMaxTM LDH Cytotoxicity

Assay Kit  utilizes the advanced WST reagent for a fast and more sensitive detection of LDH

released from damaged cells. The assay utilizing an enzymatic coupling reaction: LDH oxidizes

lactate to generate NADH, which then reacts with WST to generate yellow color. The intensity

of the generated color correlates directly with the cell number lysed. Since WST is brighter,

less amount of culture medium is required for the assay, and thus the background from serum

and culture medium is significantly reduced. Using the assay, cells can be cultured in regular

10% serum containing medium, no reducing serum or special medium is required for the

assay. In addition, since the WST is more stable, the reaction can be read multiple times, and

can also be stopped at any time point during the reaction. LDH activity can be easily quantified

by spectrophotometer or plate reader at OD450 nm. The kit provides all necessary reagents

including LDH positive control. Assay takes less than 1 hour.

II. Kit Contents:

WST Substrate Mix (CT-001-1)

LDH Assay Buffer 50 ml (CT-001-2)

Cell Lysis Solution 6 ml (CT-001-3)

Stop Solution 6 ml ( CT-001-4)

III. Preparation of Working Solutions:

• Reconstitute the WST Substrate Mix in 1 ml ddH2O for 10 min and mix thoroughly. The

solution is stable for 2 month at 4oC.

• Preparation of LDH Reaction Mix: For 100 assays, mix 200 μl of WST Substrate Mix with

10.5ml of LDH Assay Buffer. The LDH Reaction Mix should be stable for several weeks at 40C.

VI. LDH-Cytotoxicity Assay Protocol:

1. Collect cells and wash once with fresh regular culture medium, then seed 100 μl cells (with

2-10 x 104 cells*) in a 96-well plate as the following:

Background Control: 100μl culture medium per well in triplicates with no cells. The

Background Control will measure reagents and LDH background from culture

medium serum. The background value has to be subtracted from all other values.

Low Control: 100 μl cells in triplicate wells.

High Control: 100 μl cells in triplicates, add 10 μl Cell Lysis Solution each well, mix. To

adjust the increase of medium volume, 11 μl of the medium may be used in LDH

activity assay at step 4.

Test Sample: 100 μl cells in triplicates, add test substances each well, mix.

Notes:

a) The amount of cells to be used per well depends on the cell types. To optimize the

assay, you can do a quick testing by using 2, 4, 8 x 104 cells per well, then follow the

assay protocol to determine the cell number you should use. The high control should

be OD450nm ~2.0 after 30 min treatment with 10% Cell Lysis Solution, while the low

control should be OD450nm < 0.8. The reaction time should be set at ~30 min.

b) If the test substances are not dissolved in PBS, a solvent control may be performed by

addition of the same amount of solvent in triplicates without testing substances.

2. Incubate cells in an incubator (5% CO2, 90% humidity, 37oC) for the appropriate time of

treatment determined for test substance. Gently shake the plate at end of the incubation to

ensure LDH is evenly distributed in the culture medium.

3. Centrifuge cells at 600 g for 10 min to precipitate the cells.

4. Transfer the clear medium solution (10 μl/well) into an optically clear 96-well plate.

5. Add 100 μl LDH Reaction Mix to each well, mix and incubate for 30 min** at room

temperature.

6. Measure the absorbance of all controls and samples with a plate reader equipped with 450

nm (440 nm to 490 nm) filter. The reference wavelength should be 650 nm.

**Notes:

a) The reaction time can be decreased or increased depend on the color

development. The plate can be read at multiple time points until the desired reading

is observed. The high control should be OD450nm ~2.0, while the low control should

be OD450nm <0.8.

b) The reaction can be stopped by adding 10 μl of Stop Solution, mix and read within

48 hours without significant changes. Protect the reaction from light and

evaporation.

V. Calculation of the Percentage Cytotoxicity:

(Test Sample - Low Control)

Cytotoxicity (%) =-------------------------------------------  X 100

(High Control - Low Control)

For Research use only, not for clinic and diagnostic applications.