AccuRuler Ultra RGB Protein Ladder (New)

With Enhanced Wash and Striping Durability

Catalog #: PM-003, 100 loading

The Ultra Prestained Protein Ladder is a three-color protein standard with 10 pre-stained proteins covering a wide range of molecular weights from 6.5 to 270 kDa. Proteins are covalently coupled with a blue chromophore except for three reference bands (two orange bands at 30 kDa and 270 kDa and one green band at 52 kDa) when separated on SDS-PAGE (Tris-glycine buffer).  The Ultra Prestained Protein Ladder is designed for monitoring protein separation during SDS-polyacrylamide gel electrophoresis and verifying Western transfer efficiency on membranes (PVDF, nylon, or nitrocellulose) and for approximating the size of proteins. The ladder is supplied in the gel loading buffer and is ready to use.

Storage Buffer

62.5 mM Tris-H3PO4 (pH 7.5 at 25 °C), 1 mM EDTA, 2% SDS, 10 mM DTT, 1 mM NaN3, 33% glycerol.

Features:

• Sharp and stable
• Ready to use – no need to boil.
• 3 μl or 5 μl per loading for clear visualization during electrophoresis on 15-well or 10-well mini-gel, respectively
• 2 μl to 3 μl per well for general Western transferring

【Protocol】

- Thaw the product and leave it for about 10 minutes at room temperature, mix completely prior to use.

- For mini gels, take 2.5ul l of this product by micropipette or micro-syringe (wipe off liquid adhered to the outer surface of the tip) and apply it carefully into a well of the gel preferably next to the sample. For large size gel (16 x 20 cm), use 5-10 l of the product.

- Apply electric current to start electrophoresis.

【Storage】　	-20 ℃

【Expiration】　	1 year from the date of receiving.

Warranty

These products are warranted to perform as described in their labeling and in BioPioneer literature when used in accordance with their instructions. THERE ARE NO WARRANTIES THAT EXTEND BEYOND THIS EXPRESSED WARRANTY AND BIOPIONEER DISCLAIMS ANY IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR PARTICULAR PURPOSE. BioPioneer’s sole obligation and purchaser’s exclusive remedy for breach of this warranty shall be, at the option of BioPioneer Inc., to repair or replace the products. In no event shall BioPioneer Inc.  be liable for any proximate, incidental or consequential damages in connection with the products.